- D Lacth

RTL Schematic

Test Bench

D latch is given yourself when enable is 1. If enable is 0, Q is acting like memory.

| D | En | Q | Qn |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 1 |

- PIPO Register

RTL Schematic

Test Bench

PIPO Register is acting like D latch , It is given data when load is 1. If load is 0 , output will give the before result.

FOR Example : Our input is 10 and load is 1 , our output will be 10 . After that we change ınput and load will be 0 , our output doesn’t change because when load is 0 , output shows before result.

- MULTİPLEXER

RTL Schematic

Test Bench

We use multiplexer to transfer the selected value.

For example : We want to see input A . If S will be 1 , we can see it .

| INPUTS | SELECT LİNE | OUTPUT |
| --- | --- | --- |
| X | 1 | A |
| X | 0 | B |

- Logic Unit

RTL Schematic

Test Bench

We use 4 logic operations in this test bench.

- When S = 000 , A and B ( A(1) and B(1) = T(1) , A(0) and B(0) = T(0) )

- When S = 001 , A or B ( A(1) or B(1) = T(1) , A(0) or B(0) = T(0) )

- When S = 010 , Not A

- When S = 011 , A xor B ( A(1) xor B(1) = T(1) , A(0) xor B(0) = T(0) )

S(2)  doesn’t change anything.

- Arithmetic Unit

RTL Schematic

Test Bench

We use arithmetic operations in this test bench.

- When S = 000 , (A + B)

- When S = 001 , (A - B)

- When S = 010 , (A + 1)

- When S = 011 , (A >=< B) ( In comparator , I designed output like GEL so , ıf A <B , output is given 001 , If A = B , output is given 010 , If A > B , output is given 100.)

- Arithmetic Logic Unit

RTL Schematic

Test Bench

First quadruple is for Arithmetic Operation

- When S = 000  (TA + TB)

- When S = 001  (TA - TB)

- When S = 010  (TA + 1)

- When S = 011  (TA >=< TB)

Second quadruple is for Logic Operation

- When S = 100  (TA and TB)

- When S = 101  (TA or TB)

- When S = 110  (Not TA)

- When S = 111  (TA xor TB)