ELE227 FUNDAMENTALS OF DIGITAL SYSTEM LABORATORY

PRELIMINARY WORK-3

Student

Name: Egemen Can

Surname: Ayduğan

ID: 21728036

1.)

Circuit Diagram:

VHDL Implementation

| entity DECODER is Port ( A : in STD_LOGIC; B : in STD_LOGIC; C : in STD_LOGIC; D : out STD_LOGIC_VECTOR (7 downto 0)); end DECODER; architecture Behavioral of DECODER is begin D(0) <= (not A) and (not B) and (not C); D(1) <= (not A) and (not B) and C; D(2) <= (not A) and B and (not C); D(3) <= (not A) and B and C; D(4) <= A and (not B) and (not C); D(5) <= A and (not B) and C; D(6) <= A and B and (not C); D(7) <= A and B and C; end Behavioral; | entity FullSubstractorByDecoder is Port ( x : in STD_LOGIC; y : in STD_LOGIC; z : in STD_LOGIC; Dif : out STD_LOGIC; Bor : out STD_LOGIC); end FullSubstractorByDecoder; architecture Behavioral of FullSubstractorByDecoder is component DECODER Port ( A : in STD_LOGIC; B : in STD_LOGIC; C : in STD_LOGIC; D : out STD_LOGIC_VECTOR (7 downto 0)); end component; signal T : STD_LOGIC_VECTOR (7 downto 0); begin Dec1 : DECODER port map(x,y,z,T(7 downto 0)); Dif <= T(1) or T(2) or T(4) or T(7); Bor <= T(1) or T(2) or T(3) or T(7); end Behavioral; |
| --- | --- |

RLT Schematic:

stim_proc: process

begin

x <='0';

y <='0';

z <='0';

wait for 100 ns;

x <='0';

y <='0';

z <='1';

wait for 100 ns;

x <='0';

y <='1';

z <='0';

wait for 100 ns;

x <='0';

y <='1';

z <='1';

wait for 100 ns;

x <='1';

y <='0';

z <='0';

wait for 100 ns;

x <='1';

y <='0';

z <='1';

wait for 100 ns;

x <='1';

y <='1';

z <='0';

wait for 100 ns;

x <='1';

y <='1';

z <='1';

wait;

end process;

TEST BENCH INPUT:

TEST BENCH RESULTS:

2.)

Circuit Diagram:

VHDL Implementation

| entity FourXOneMux is Port ( I : in STD_LOGIC_VECTOR (3 downto 0); S : in STD_LOGIC_VECTOR (1 downto 0); Y : out STD_LOGIC); end FourXOneMux; architecture Behavioral of FourXOneMux is begin Y <= ((not S(1)) and (not S(0)) and I(0)) or ((not S(1)) and S(0) and I(1)) or (S(1) and (not S(0)) and I(2)) or (S(1) and S(0) and I(3)); end Behavioral; | entity Fonkby4x1Mux is Port ( x : in STD_LOGIC; y : in STD_LOGIC; z : in STD_LOGIC; F : out STD_LOGIC); end Fonkby4x1Mux; architecture Behavioral of Fonkby4x1Mux is component FourXOneMux Port ( I : in STD_LOGIC_VECTOR (3 downto 0); S : in STD_LOGIC_VECTOR (1 downto 0); Y : out STD_LOGIC); end component; signal T : STD_LOGIC_VECTOR (3 downto 0); signal K : STD_LOGIC_VECTOR (1 downto 0); begin T(0) <= '1'; T(1) <= z; T(2) <= '1'; T(3) <= '0'; K(0) <= y; K(1) <= x; M1: FourXOneMux port map (T(3 downto 0),K(1 downto 0),F); end Behavioral; |
| --- | --- |

RLT Schematic:

stim_proc: process

begin

x<='0';

y<='0';

z<='0';

wait for 100 ns;

x<='0';

y<='0';

z<='1';

wait for 100 ns;

x<='0';

y<='1';

z<='0';

wait for 100 ns;

x<='0';

y<='1';

z<='1';

wait for 100 ns;

x<='1';

y<='0';

z<='0';

wait for 100 ns;

x<='1';

y<='0';

z<='1';

wait for 100 ns;

x<='1';

y<='1';

z<='0';

wait for 100 ns;

x<='1';

y<='1';

z<='1';

wait;

end process;

TEST BENCH INPUT:

TEST BENCH RESULTS:

3.)

Circuit Diagram:

VHDL Implementation:

| entity decoder24 is Port ( A : in STD_LOGIC; B : in STD_LOGIC; D : out STD_LOGIC_VECTOR (3 downto 0)); end decoder24; architecture Behavioral of decoder24 is begin D(0) <= (not A) and (not B); D(1) <= (not A) and B; D(2) <= A and (not B); D(3) <= A and B; end Behavioral; | entity COMP24 is Port ( A : in STD_LOGIC_VECTOR (1 downto 0); B : in STD_LOGIC_VECTOR (1 downto 0); G : out STD_LOGIC; E : out STD_LOGIC; L : out STD_LOGIC); end COMP24; architecture Behavioral of COMP24 is COMPONENT decoder24 Port ( A : in STD_LOGIC; B : in STD_LOGIC; D : out STD_LOGIC_VECTOR (3 downto 0)); end COMPONENT; signal T : STD_LOGIC_VECTOR (7 downto 0); begin D1 : decoder24 port map(A(0),B(0),T(3 downto 0)); D2 : decoder24 port map(A(1),B(1),T(7 downto 4)); G <= T(6) or ((T(7) or T(4)) and T(2)) ; L <= T(5) or ((T(7) or T(4)) and T(1)) ; E <= (T(7) or T(4)) and (T(3) or T(0)); end Behavioral; |
| --- | --- |

RLT Schematic:

stim_proc: process

begin

A <= "00";

B <= "00";

wait for 100 ns;

A <= "00";

B <= "10";

wait for 100 ns;

A <= "00";

B <= "11";

wait for 100 ns;

A <= "01";

B <= "00";

wait for 100 ns;

A <= "01";

B <= "01";

wait for 100 ns;

A <= "10";

B <= "01";

wait for 100 ns;

A <= "10";

B <= "10";

wait for 100 ns;

A <= "10";

B <= "11";

wait for 100 ns;

A <= "11";

B <= "10";

wait for 100 ns;

A <= "11";

B <= "11";

wait;

end process;

TEST BENCH INPUT:

TEST BENCH RESULTS:

4.)

Circuit Diagram:

VHDL Implementation

| entity ShiftAdder is Port ( A : in STD_LOGIC_VECTOR (3 downto 0); S : out STD_LOGIC_VECTOR (3 downto 0)); end ShiftAdder; architecture Behavioral of ShiftAdder is begin S(0)<= ((not A(3)) and (not A(2)) and A(0))or ((not A(3)) and A(2) and A(1) and (not A(0)))or (A(3) and (not A(2)) and (not A(1)) and (not A(0))); S(1)<= ((not A(3)) and (not A(2)) and A(1))or ((not A(3)) and A(1) and A(0))or (A(3) and (not A(2)) and (not A(1)) and (not A(0))); S(2)<= (A(3) and (not A(2)) and (not A(1)) and A(0))or ((not A(3)) and A(2) and (not A(1)) and (not A(0))); S(3)<= ((not A(3)) and A(2) and A(0))or ((not A(3)) and A(2) and A(1))or (A(3) and (not A(2)) and (not A(1))); end Behavioral; | entity Converter is Port ( A : in STD_LOGIC_VECTOR (5 downto 0); B : out STD_LOGIC_VECTOR (6 downto 0)); end Converter; architecture Behavioral of Converter is component ShiftAdder Port ( A : in STD_LOGIC_VECTOR (3 downto 0); S : out STD_LOGIC_VECTOR (3 downto 0)); end component; signal T : STD_LOGIC_VECTOR (9 downto 0); signal K : STD_LOGIC_VECTOR (3 downto 0); begin K(3)<= '0'; K(2)<= A(5); K(1)<= A(4); K(0)<= A(3); B(6)<= T(9); T(5)<= A(2); B(5)<= T(4); T(0)<= A(1); SA1 : ShiftAdder port map (K(3 downto 0),T(9 downto 6)); SA2 : ShiftAdder port map (T(8 downto 5),T(4 downto 1)); SA3 : ShiftAdder port map (T(3 downto 0),B(4 downto 1)); B(0) <= A(0); end Behavioral; |
| --- | --- |

RLT Schematic:

stim_proc: process

begin

A <= "101010";

wait for 100 ns;

A <= "010101";

wait for 100 ns;

A <= "111111";

wait for 100 ns;

A <= "111110";

wait for 100 ns;

A <= "010110";

wait for 100 ns;

A <= "010001";

wait for 100 ns;

A <= "011011";

wait for 100 ns;

A <= "101101";

wait;

end process;

TEST BENCH INPUT:

TEST BENCH RESULTS: