HACETTEPE UNIVERSITY

ELECTRICAL AND ELECTRONICS ENGINEERING

ELE227 FUNDAMENTALS OF DIGITAL SYSTEMS LABORATORY
FALL 2019

Egemen Can AYDUĞAN - 21728036

1-A) S-R Latch Circuit Diagram

VHDL Implementation:

entity SRLatch is

Port ( S : in  STD_LOGIC;

R : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end SRLatch;

architecture Behavioral of SRLatch is

signal T : STD_LOGIC_VECTOR(1 downto 0);

begin

Q <= S nand T(1);

Qn <= R nand T(0);

T(0) <= S nand T(1);

T(1) <= R nand T(0);

end Behavioral;

RTL Schematic:

1-B) D Latch

Circuit Diagram

VHDL Implementation:

entity DLatch is

Port ( D : in  STD_LOGIC;

En : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end DLatch;

architecture Behavioral of DLatch is

component SRLatch

Port ( S : in  STD_LOGIC;

R : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end component;

signal K : STD_LOGIC_VECTOR(1 downto 0);

begin

K(0) <= D nand En;

K(1) <= (not D) nand En;

C : SRLatch port map (K(0),K(1),Q,Qn);

end Behavioral;

RTL Schematic:

1-C) Positive Edge D Flip-Flop

Circuit Diagram

VHDL Implementation:

entity DFlipFlop is

Port ( D : in  STD_LOGIC;

Clk : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end DFlipFlop;

architecture Behavioral of DFlipFlop is

component DLatch

Port ( D : in  STD_LOGIC;

En : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end component;

signal T : STD_LOGIC_VECTOR(1 downto 0);

begin

C1 : DLatch port map(D,Clk,T(0),T(1));

C2 : DLatch port map(T(0),(not Clk),Q,Qn);

end Behavioral;

RTL Schematic:

1-D) Negative Edge T Flip-Flop

Circuit Diagram

VHDL Implementation:

entity NegTFlipFlop is

Port ( T : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clk : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end NegTFlipFlop;

architecture Behavioral of NegTFlipFlop is

component JKFlipFlop

Port ( J : in  STD_LOGIC;

K : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clk : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end component;

begin

C : JKFlipFlop port map(T,T,Reset,(not Clk),Q,Qn);

end Behavioral;

RTL Schematic:

2) Circuit Diagram

VHDL Implementation:

entity SequenceDetector is

Port ( X : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clk : in  STD_LOGIC;

Y : out  STD_LOGIC);

end SequenceDetector;

architecture Behavioral of SequenceDetector is

component JKFlipFlop

Port ( J : in  STD_LOGIC;

K : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clk : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end component;

signal JA,JB,JC,KA,KB,KC : STD_LOGIC;

signal A,B,C : STD_LOGIC;

begin

JA <= B and C and x;

KA <= '1';

JB <= (A and (not x)) or (C and (not x));

KB <= x or C;

JC <= x or B;

KC <= (not x) or B;

FF1 : JKFlipFlop port map(JA,KA,Reset,Clk,A);

FF2 : JKFlipFlop port map(JB,KB,Reset,Clk,B);

FF3 : JKFlipFlop port map(JC,KC,Reset,Clk,C);

Y <= A ;

end Behavioral;

RTL Schematic:

Test Bench Input

constant Clk_period : time := 100 ns;

Clk_process :process

begin

Clk <= '1';

wait for Clk_period/2;

Clk <= '0';

wait for Clk_period/2;

end process;

stim_proc: process

begin

-- hold reset state for 100 ns.

Reset <= '1';

wait for 100 ns;

x <= '1';

Reset <= '0';

wait for 100 ns;

x <= '0';

wait for 100 ns;

x <= '0';

wait for 100 ns;

x <= '1';

wait for 100 ns;

x <= '0';

wait for 100 ns;

x <= '0';

wait for 100 ns;

x <= '1';

wait;

end process;

Test Bench Result

3) Circuit Diagram

VHDL Implementation:

entity ODDParity is

Port ( D : in  STD_LOGIC_VECTOR (11 downto 0);

Pin : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clock : in  STD_LOGIC;

Pout : out  STD_LOGIC;

Pche : out  STD_LOGIC);

end ODDParity;

architecture Behavioral of ODDParity is

component TFF

Port ( T : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clock : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end component;

signal T1,T2,T3 : STD_LOGIC;

begin

T1 <= D(0) xor D(1) xor D(2) xor D(3) xor

D(4) xor D(5) xor D(6) xor D(7) xor

D(8) xor D(9) xor D(10) xor D(11) ;

FF : TFF port map(T1,Reset,Clock,T2,T3);

Pout <= T2;

Pche <= Pin xnor T2;

end Behavioral;

RTL Schematic:

Test Bench Input

constant Clock_period : time := 100 ns;

Clock_process :process

begin

Clock <= '1';

wait for Clock_period/2;

Clock <= '0';

wait for Clock_period/2;

end process;

begin

Reset <= '1';

wait for 100 ns;

D <= "101101010110";

Pin <= '1';

Reset <='0';

wait for 100 ns;

D <= "111100101101";

Pin <= '1';

wait for 100 ns;

D <= "000000001010";

Pin <= '0';

wait for 100 ns;

D <= "111111111111";

Pin <= '0';

wait;

end process;

Test Bench Result

4) Circuit Diagram

VHDL Implementation:

entity JFF is

Port ( J : in  STD_LOGIC;

K : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clock : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end JFF;

architecture Behavioral of JFF is

component DFF

Port ( D : in  STD_LOGIC;

Reset : in  STD_LOGIC;

Clock : in  STD_LOGIC;

Q : out  STD_LOGIC;

Qn : out  STD_LOGIC);

end component;

signal T1,T2,T3,T4,T5 : STD_LOGIC;

begin

T1 <= J and T5;

T2 <= (not K) and T4;

T3 <= T1 or T2;

FF : DFF port map (T3,Reset,Clock,T4,T5);

Q <= T4;

Qn <= T5;

end Behavioral;

RTL Schematic:

Test Bench Input

constant Clock_period : time := 100 ns;

-- Clock process definitions

Clock_process :process

begin

Clock <= '0';

wait for Clock_period/2;

Clock <= '1';

wait for Clock_period/2;

end process;

stim_proc: process

begin

Reset <= '1';

wait for 100 ns;

J <= '0';

K <= '0';

Reset <= '0';

wait for 100 ns;

J <= '0';

K <= '1';

wait for 100 ns;

J <= '1';

K <= '0';

wait for 100 ns;

J <= '1';

K <= '1';

wait;

end process;

Test Bench Result